This document is to be used with the TINA-TI program (www.ti.com/tool/TINA-TI) loaded with CP-80.TSC spice file from the TINA Spice Files Folder.

The CP-80 Spice file is a slightly modified version of the CP-80 actual preamp schematic. As you can see, the 3 position Brilliance switch is set up to manually open and close each contact.

Before we go much further, we need to talk about JFETs, their extremely wide parameter set (i.e. Very large part to part variations), and a couple definitions.

IDSS (zero-gate-voltage drain current) is a characteristic that varies between individual devices, even those of the same part number. However, a JFET's characteristics are determined by its IDSS value, and this value, along with the pinch-off voltage (VGS(off)), can be used to classify or group devices for specific applications like voltage-controlled resistors or current sources. Devices are primarily classified by their type, either N-channel or P-channel, which dictates the polarity of the gate-source voltage required to control the drain current.

How JFETs are classified and characterized

- Channel Type: The most fundamental classification is the channel type, which can be either N-channel or P-channel.

  - N-channel: Requires a negative gate-source voltage to decrease drain current.

  - P-channel: Requires a positive gate-source voltage to decrease drain current.

- IDSS and VGS(off): Within a channel type, devices are not classified by IDSS but can be grouped based on IDSS and VGS(off)values, which are measured through testing.

  - IDSS (Zero-gate-voltage drain current): The drain-source current when the gate-source voltage (VGS) is zero. This is a measure of the maximum current a JFET can conduct at VGS=0.

  - VGS(off) (Pinch-off voltage): The gate-source voltage required to turn the JFET off (reduce the drain current to near zero).

  - These two parameters are directly related, and a higher IDSS will have a different corresponding VGS(off) value than a lower IDSS.

Applications of using IDSS and VGS(off) for grouping

- Matching: Devices can be grouped or "matched" by similar IDSS and VGS(off) values for applications where this is critical, such as stereo audio amplifiers or balanced circuits.

- Current Source: A JFET with a specific IDSS can be used to create a current source or sink. By adding a resistor from the source to ground, the current can be set to a specific value, and the IDSS value of the device can be used to calculate the appropriate resistor value.

- Voltage Controlled Resistor: In a voltage-controlled resistor application, the IDSS value is used in conjunction with VGS(off) to select devices that have the same "on-resistance" (RDS(on)) range.